[image: image2.wmf]
TECHNICAL DATA FOR EL DRIVING INVERTER

	MODEL NO.
	 50450B-652Z
	ISSUED DATE:
	DEC. 31, 1997

	1. Electrical characteristics

	
	Item
	SYM
	Unit
	Test condition
	Ratings.
	

	
	Oscillation Min. Voltage
	Vin
	VDC
	
	2.0 Min.
	

	
	I/P CURRENT
	Iin
	mA
	Vin = 5.0VDC
	70 Typ.
	

	
	O/P VOLTAGE
	Vout
	Vrms
	Vin = 5.0VDC
	85 – 120
	

	
	O/P FREQUENCY
	fosc
	Hz
	Vin = 5.0VDC
	480 – 720
	

	  Vin=5V±10%, Ta=25oC, applied load area ( 70 cm2
2. Absolute maximum ratings

	
	Item
	SYM
	Min.
	Max.
	Unit
	

	
	INPUT VOLTAGE
	Vin
	0
	6.0
	VDC
	

	
	OPERATING TEMP.
	Top
	0
	50
	oC
	

	
	STORAGE TEMP.
	Tstg
	-20
	65
	oC
	

	3.Outline dimension ( in mm):


[image: image1.wmf]
Package Material : ABS Resin, Color : Black, Internal: Epoxy Resin Compound
Note:

a. Breakdown may occur if power is applied without EL lamp connected to load side.

b. Operation with the output short-circuited may cause breakdown.

c. Never exceed input voltage specification.


	DOC. NO.
	WAC-9712006
	ISSUED BY:
	REVISION
	DESCRIPTION

	DRAWN   :
	
	
	
	

	CHECKED :
	
	
	
	

	APPROVED:
	
	
	SHEET    1    OF     1


1
Total Power Electronics Incorporation


      Tel: +886(02) 8665-4110 Fax: +886(02) 8665-4580 


  Tel: (02) 2392-1184    











_961413055.doc
[image: image1.png]——30.6 Max. ——

H

H

- 15004050
3 - 9065
ignal
1] ouT
2 | OND
5] 4BV








